THE UNIVERSITY OF AZAD JAMMU & KASHMIR, MUZAFFARABAD

| COURSE TITLE | COMPUTER NETWORKS |
| --- | --- |
| COURSE CODE | CS-2203 |
| TITLE | LAB 6 |
| STUDENT NAME | HASEEB SHEIKH |
| STUDENT ROLL NUMBER | 2024-SE-31 |
| INSTRUCTOR NAME | ENGR. DR. ASMA JAVED |
| SUBMISSION DATE | SEPTEMBER 20, 2026 |

DEPARTMENT OF SOFTWARE ENGINEERING

Computer Networking

LAB 6 (Switch Communication Between Two LANs)

- Objective:

The objective of this lab is to establish communication between two separate Local Area Networks (LANs). Since switches can only communicate within the same network, a central router is configured to route traffic between the different IP subnets.

- Network Topology:

Explanation:

The network consists of a central router connecting two distinct LANs. LAN 1 operates on the 192.168.1.0      network, and LAN 2 operates on the 192.168.2.0 network.

- IP Configuration:

| Device (Network) | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC1 (LAN 1) | FastEthernet0 | 192.168.1.2 | 255.255.255.0 | 192.168.1.1 |
| PC2 (LAN 1) | FastEthernet0 | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 |
| PC3 (LAN 2) | FastEthernet0 | 192.168.2.2 | 255.255.255.0 | 192.168.2.1 |
| PC4 (LAN 2) | FastEthernet0 | 192.168.2.3 | 255.255.255.0 | 192.168.2.1 |
| Router (LAN 1 Link) | GigabitEthernet0/0/0 | 192.168.1.1 | 255.255.255.0 | N/A |
| Router (LAN 2 Link) | GigabitEthernet0/0/1 | 192.168.2.1 | 255.255.255.0 | N/A |

4. Router CLI Configuration:

Explanation:

The router's interfaces were manually assigned IP addresses that serve as the default gateways for their respective LANs. The no shutdown command was executed to activate the physical interfaces.

5. Connectivity Verification

Explanation:

A ping test was initiated from a device on LAN 1 to a device on LAN 2. The successful replies confirm that the router is correctly receiving the packets from the first subnet and routing them to the second subnet.